# Representation of GD&T annotations in the context of STEP AP210 package models

#### James Stori, SFM Technology, Inc.
September 29, 2011

The ability to represent geometric dimension and tolerance (GD&T) annotations is critical to the successful communication and archival representation of product manufacturing information (PMI) in data models based on the STEP (ISO-10303) standards. AP210 (ISO 10303-210) is the application protocol of STEP that addresses electronic assembly interconnect and packaging design. To support the ongoing development of the AP210 standard, and promote its adoption for the neutral representation of electronic component data, recent updates to the ISO-10303 standards related to the representation of GD&T annotations have been exercised in the context of AP210 package models. This document describes the detailed representation of many of the most common GD&T annotations applied to AP210 package models. The largest family of standards for packaged electronic components are those published by JEDEC. The JEDEC standards are freely and publicly available (www.jedec.org), and contain detailed dimensioned and toleranced specifications for physical package configurations. JEDEC Publication 95 is a series of documents containing specifications for many common physical package configurations. A series of 10 detailed package models spanning 5 different JEDEC package families have been created. These representative package models contains detailed three-dimensional representations, explicit representation of critical package features such as individual terminals, and GD&T annotations consistent with the JEDEC specifications. Specific examples from these representative models are used in this document to illustrate the mapping of the various concepts to STEP entities and instances in physical files. The implementation and examples span both single-solid and multiple-solid geometric representations of packages, and the explicit links between the geometry and GD&T elements. Although the immediate focus of this document is the representation of STEP GD&T concepts in AP210 package model representations, the representation of the GD&T annotations and three-dimensional geometry is common to other STEP application protocols, and these implementations and examples are applicable in the broader context of the STEP standards.

The implementation described here is based on a draft of STEP Part 47: Integrated generic resource: Shape variation tolerances [ISO 10303-47:2011-04(E)] dated April 22, 2011, and the MIM long form schema of the AP210 standard as of July 27, 2011. In addition, the GD&T representations described below are intended to be consistent with Version 3.1 of the CAX-IF Recommended Practices for the Representation and Presentation of Product Manufacturing Information (PMI). This document is intended to augment the existing recommended practices with a specific focus on the representation of GD&T in the context of AP210 package models, as well as to illustrate a variety of specific GD&T concepts through detailed representative examples. Representative examples from two of the generated physical files are used in the discussion below to illustrate a number of the most commonly occurring GD&T concepts in the representation of JEDEC package models. Examples are taken from both a JEDEC transistor package family, TO-236 and a diode package family DO-214. The JEDEC drawing containing the GD&T annotations as well as a rendering of the corresponding 3D package models is contained in Appendix Figures A1 and A2.

## Package Model Representation

Figure 1 illustrates some of the most important top level elements in the AP210 package model. Each of the AP210 models contains a single package and a detailed description of its physical representation. The package is referenced by a package_body and typically multiple package_terminal. Each package may contain multiple shape_representations. For the purpose of the GD&T annotations, we are focused on the three-dimensional shape representation whose purpose is identified as ‘design’. This shape_representation will be related to the package as shown in Figure 1, and may be identified through the combination of the associated description_attribute and the related representation with name ‘predefined_shape_purpose’. There are two common representations of 3D geometric models supported in the examples provided. The 3D geometry of a package may be represented as either a single solid or as an assembly-based representation. In either case, the geometric representation of the individual terminals and/or body of the package must be associated with the corresponding package_body and package_terminals. In the preferred representation, each terminal and/or body contains its own three-dimensional representation (most commonly an advanced_brep_shape_representation) and these shape representations are mapped into the shape representation of the package with a usage_concept_usage_relationship. In this scenario, many similar terminals of a package can share a single package_terminal_template_definition, which in turn, has an associated advanced_brep_shape_representation. Similarly, the body of the package can have its own associated three-dimensional shape representation, which is mapped into the shape representation of the package with a usage_concept_usage_relationship. The resulting 3d shape representation of the package is analogous to a mechanical assembly composed of the terminals and body. This representation is preferred as it is more geometrically efficient, and more directly supports processing of the package model by downstream applications that must manipulate, use, and/or analyze the shapes of the individual package features.

Figures 2 and 3 illustrate the representation of a package_terminal and package_body respectively in the ‘assembly-style’ package structure, in which each has a 3d shape represented by an advanced_brep_shape_representation. Note that in Figure 2, multiple instances of package_terminal will often share a single package_terminal_template_definition and each terminal template definition will have its own associated shape_representation. A single terminal template definition would be used by multiple terminals sharing a single geometric model, and other common attributes such as lead form. For example, if a given package has 64 similar gullwing terminals, and a single thermal pad, there would be a total of 65 instances of package_terminal and two terminal template definitions, each with its own shape representation. For each individual terminal, the usage_concept_usage_relationship (a subtype of mapped_item) positions the terminal within the shape representation of the package. The mapping_target and mapping_source attributes specify the geometric placement operation for the terminal.

The alternative common geometric representation of a package model would be based on a single geometric model, most commonly a single manifold_solid_brep within a single shape_representation. In that scenario, the relationship between the individual package terminals and package body and their associated geometry in the shape representation of the package would typically be expressed with the mechanism of the item_identified_representation_usage (or a relevant subtype). This would most commonly result in a mapping of groups of faces in the shape representation of the package to each of the terminals and body. This representation presents challenges for applications that must leverage and/or identify common geometry across similar terminals. Nevertheless, it does simplify several aspects of the geometric representation, and would be commonly encountered in 3D models provided by component suppliers, such as connector vendors. Each of the common package model representations (single vs. multiple solid) requires a specific mechanism in STEP for associating faces of the geometric model with the applicable GD&T elements.

## Shape_aspects used in the representation of GD&T annotations

The shape_aspect entity is used to represent a wide variety of features in STEP product models. Both package_terminal and package_body are subtypes of shape_aspect. Additionally, many of the entities used to represent specific GD&T concepts, including datum, datum_feature, centre_of_symmetry, etc., are subtypes of shape_aspect. Multiple shape_aspects will typically be used in the representation of an individual dimension or tolerance. At the lowest level, it is desired to link the GD&T elements to one or more faces in the applicable underlying solid model or models. There are two specific mechanisms that may be used to relate a shape aspect to a face in the model.

In the case that the package is represented by a single solid, a geometric_item_specific_usage may be used to relate a single face of a shape_representation with the shape_aspect.Figures 4 illustrates the instantiation of a shape_aspect used to identify a single face in the context of a single solid package model. It is important to note that in the single-solid package model, a face used in a GD&T annotation will also typically be an element of either the package_body or a package_terminal. If it is desired to identify the individual terminal or terminals, for example, referenced by a GD&T element, the receiving system must traverse the relationships from the shape aspects composing the GD&T element through the associated face or faces and back to any additional package terminals referencing the common face(s).

In the context of a multiple-solid (assembly-style) package representation, the geometric_item_specific_usage is insufficient to identify individual faces of individual terminals in the model. This is due to the fact that many terminal instances will often be sharing a single shape_representation, and therefore a common collection of faces. In this case, a specialized subtype of item_identified_representation_usage must be employed. The chain_based_geometric_item_specific_usage enables the identification of a single representation_item, such as an advanced_face, at the leaf node of a chain of mapped representations. The entities and relationships needed to uniquely identify a single face in the context of a multiple-solid package model representation are illustrated in Figures 2 and 3. Figure 2 illustrates a shape_aspect (#1020) calling out a single face (#670) on a single instance of a package_terminal.  The chain_based_geometric_item_specific_usage contains two nodes – the first element in the list of nodes references the shape_representation of the package (#816) while the second element references the shape_representation of the terminal (#784). The undirected_link list contains a single element, the usage_concept_usage_relationship (a subtype of mapped_item) that places an instance of the terminal shape representation into the package shape representation for one specific package_terminal. Per the recommended practices, if a single geometric feature consists of multiple faces, it is appropriate to have multiple item_identified_representation_usage entities relating the individual faces to the single shape aspect. This will typically be the case for the representation of the package terminal or body, and would also be applicable for certain GD&T features, such as a hole consisting of two cylindrical faces.

#### Composite_shape_aspect

Often, it is either necessary or convenient to represent a shape_aspect that is a composition of other existing shape_aspects. A composite_group_shape_aspect and a composite_unit_shape_aspect are two important subtypes of composite_shape_aspect. A composite_unit_shape_aspect is used to aggregate multiple shape_aspects that are to be treated as a unit.  The composite_group_shape_aspect is used when it is desired to apply a property to each of the constituent elements individually. When representing other specialized GD&T subtypes of shape_aspect, a complex with composite_shape_aspect can also be instantiated. Figures 5 and 6 below illustrate several of the possible scenarios involving composite_shape_aspect.

Figure 5, below details the representation of what would commonly be referred to as a ‘regular feature of size’ – two opposed parallel surfaces – that define a length measure that is independent of the location of the feature (a dimensional_size). In this case, the same feature of size serves as a datum_feature for this particular package. Finally, this feature is a composite_shape_aspect, composed of the shape aspects representing each of the opposed parallel surfaces. As a result, a complex composite_shape_aspect+dimensional_size_with_datum_feature (#1646) has been instantiated. Also important to note in this example is the fact that each of the two opposed parallel surfaces is split by the presence of the terminal. Each of these two shape aspects is represented as a ‘single geometric feature’ with relationships to both of the associated faces in the model. In this particular example (single-solid package), there would be two geometric_item_specific_usage instances for each shape aspect.

Figure 6 illustrates the instantiation of a composite_unit_shape_aspect. In this case, a datum_feature is composed of the bottom faces of each of the three terminals in the package. The datum_feature is also a tangent. The composite_unit_shape_aspect is appropriate in this scenario, as the three faces taken together are to be treated as a unit. As this model is represented with an assembly-based structure, each of the three single-face shape aspects (#1018, #1020, #1022) is linked to a single face in the shape representation of the terminal template definition through a chain_based_geometric_item_specific_usage. With reference to the sample physical file provided (TO-236_AA_with_gdt.stp), each of these three shape aspects is actually calling out an individual placement of the same face (#670) – through the usage_concept_usage_relationship referenced by the chain_based_geometric_item_specific_usage.

#### Centre_of_symmetry and tangent

Both center planes and tangent planes are commonly occurring elements in GD&T representation. Both are represented as subtypes of derived_shape_aspect. A center plane is represented in STEP as a centre_of_symmetry that is ‘derived from’ a symmetric_shape_aspect, while a tangent may be derived from either a simple shape_aspect or a composite_shape_aspect. In both cases, the geometry of the derived plane will be represented as a plane in a constructive_geometry_representation associated with the derived_shape_aspect. In the case of the center plane, the symmetric_shape_aspect will most commonly be two parallel opposing surfaces. If each of these is represented as a shape_aspect, the, symmetric_shape_aspect will also be a composite_shape_aspect. Figure 7 illustrates the representation of a center plane for a single terminal of the package, and the associated symmetric_shape_aspect representing the feature of size corresponding to the width of the terminal. Note the presence of the associated plane in a constructive_geometry_representation. This constructive_geometry_representation is related to the shape_representation of the package.

## Datum and Datum_system

A datum in STEP is most commonly established by a datum_feature or a set of datum_targets. The datums in JEDEC package models have all been established based on a datum feature. A package model will typically have a datum coincident with the ‘seating plane’ of the component. In the JEDEC specifications, by convention, this is usally Datum C. Figure 6 illustrates the representation of Datum C in the TO-236 package. The datum is related to a single datum_feature that is also a tangent. The tangent is ‘derived from’ a composite_unit_shape_aspect, which consists of the faces tangent to the seating plane (one from each terminal). The orientation of the tangent is expressed by the associated plane in the constructive_geometry_representation. In the JEDEC specifications, datums A and B are typically orthogonal planes passing through the center of the package. The corresponding datum feature usually consists of opposing parallel sides of the package body. These datum features are expressed as a complex of dimensional_size_with_datum_feature and composite_shape_aspect. With reference to Figure A2, the datum feature for datum B is represented as in Figure 5. The composite_shape_aspect is composed of the opposing parallel surfaces. The fact that this same shape aspect is a dimensional_size enables simultaneous representation of dimension E1. The representation of a datum_system will be discussed below in the context of geometric_tolerance.

## Dimensional_size

A dimensional_size is used to assign a length measure to a ‘feature of size’. Many of the common critical JEDEC package dimensions, such as toe-to-toe spans, body length and width, terminal width, etc. are best represented as a dimensional_size. A dimensional_size is associated with a shape_dimension_representation that contains the specific value or values of the dimension.  In the generated examples, all shape_dimension_representation contain one or more values, qualified by a type_qualifier. The implementation supports type qualifier values of ‘minimum’, ‘average’, and ‘maximum’, although additional qualifications are possible. There is no requirement or expectation that all qualified values will be provided for a given dimension. Often, the JEDEC specifications will provide all three values. In certain instances, however, only a minimum (or maximum) value may be available, or both minimum and maximum values are provided, but there is no nominal (or average) value available. Figure 8 illustrates the representation of the dimensional_size and corresponding shape_dimension_representation for dimension E1 in the DO-214_AB package.  There are three qualified values for this dimension (minimum, average, and maximum), each represented by a complex length_measure_with_unit+measure_representation_item+ qualified_representation_item.

## Dimensional_location

A dimensional_location is used to associate a length measure with two shape_aspects as a dimensional measure. In fact, the dimensional_location is a subtype of a shape_aspect_relationship. In the DO-214_AB example, there are several dimensions populated as a dimensional_location, such as dimensions A, A2, and A3 (see Figure A2). In these three examples, the dimension is a length measure between datum feature C and one of the parallel surfaces on the body of the package. Figure 9 illustrates a dimensional_location and corresponding shape_dimension_representation for dimension A (the overall package height) in the DO-214_AB package. The representation of the value of the dimension, through the use of one or more qualified_representation_item is identical to the case of the dimensional_size.

## Geometric_tolerance

STEP supports the representation of specific tolerance types through subtypes of geometric_tolerance. The JEDEC package models contain specific examples of several of these tolerance representations, including position_tolerance, surface_profile_tolerance, and parallelism_tolerance. Many times, the specific tolerance type will be instantiated as a complex with geometric_tolerance_with_datum_reference and/or modified_geometric_tolerance, as described in the recommended practices. Figures 10 and 11 demonstrate the population of a position tolerance with both a reference to a datum system and a tolerance modifier (least material condition). This particular tolerance controls the position of the center plane of each of the terminals independently, with respect to the established datum system. For this reason, the tolerance is associated with a composite_group_shape_aspect, which is in turn composed of a centre_of_symmetry for each of the two terminals in the package. A variety of other representational examples of geometric tolerances are contained in the populated physical files.

Figure 1. Top-level elements in the package model and the three-dimensional shape_representation.

Figure 2. A terminal, associated template, and single face shape_aspect  in an ‘assembly’ style representation.

Figure 3. The package body and an associated single face shape_aspect in an ‘assembly’ style representation.

Figure 4. Associations between a single face and a shape aspect of the package in a single-solid representation. A single face may be related to a terminal or the package body as well as a shape_aspect contributing to a GD&T annotation.

Figure 5. Datum B and Dimension E1 – DO-214_AB (single solid).

#1594=SHAPE_ASPECT('single geometric feature','gdt body length f2',#3,.T.);

#1628=SHAPE_ASPECT('single geometric feature','gdt body length f1',#3,.T.);

#1646=(COMPOSITE_SHAPE_ASPECT()DATUM_FEATURE()DIMENSIONAL_SIZE(#1646,'length')DIMENSIONAL_SIZE_WITH_DATUM_FEATURE()SHAPE_ASPECT('feature of size','dimension E1 and datum feature B',#3,.T.));

Figure 6. Datum C – TO-236_AA (TO-236_AA_with_gdt.stp).

#1020

#1018

#1022=SHAPE_ASPECT('single element','gdt terminal bottom pin 3',#3,.T.);

#1086=COMPOSITE_UNIT_SHAPE_ASPECT('multiple elements','bottom all pins',#3,.T.);

Figure 7. Representation of a center plane – TO-236_AA (TO-236_AA_with_gdt.stp).

#1032

#1040

#1134=CENTRE_OF_SYMMETRY('centre plane','pin 1 centre_of_symmetry',#3,.F.);

#1131=(COMPOSITE_SHAPE_ASPECT()SHAPE_ASPECT('feature of size','pin 1 width feature of size',#3,.T.) SYMMETRIC_SHAPE_ASPECT());

Figure 8. Dimension E1 – DO-214_AB (single solid).

#1594=SHAPE_ASPECT('single geometric feature','gdt body length f2',#3,.T.);

#1628=SHAPE_ASPECT('single geometric feature','gdt body length f1',#3,.T.);

#1646=(COMPOSITE_SHAPE_ASPECT()DATUM_FEATURE()DIMENSIONAL_SIZE(#1646,'length')DIMENSIONAL_SIZE_WITH_DATUM_FEATURE()SHAPE_ASPECT('feature of size','dimension E1 and datum feature B',#3,.T.));

Figure 9. Dimension A – DO-214_AB (single solid).

#1662=(DATUM_FEATURE()DERIVED_SHAPE_ASPECT()SHAPE_ASPECT('tangent plane','datum feature C',#3,.T.)TANGENT());

#1626=SHAPE_ASPECT('single element','gdt body top',#3,.T.);

#1769=DIMENSIONAL_LOCATION('linear distance',$,#1626,#1662);B',#3,.T.));

Figure 10. Position_tolerance and datum_system – DO-214_AB (single solid).

Figure 11. Terminal center plane position_tolerance – DO-214_AB (single solid).

#### APPENDIX A– Generated Physical Files

AP210 package models have been generated and GD&T annotations have been added for instances from 5 separate package families. The examples were chosen to illustrate a broad range of package styles. Four of the five families are based on the JEDEC Publication 95 mechanical outlines. The fifth is a representative example of a common ‘chip’ style package not addressed by JEDEC. For each of the five package models, two separate representations were instantiated –both a single solid representation and a multiple-solid representation.

The table below summarizes the families and instances represented:

| Package | Category / Leadform | Image | AP210 models |
| --- | --- | --- | --- |
| DO-214_AB | DO / Ribbon L - Inward |  | DO-214_AB_with_gdt.stp DO-214_AB_with_gdt_single_solid.stp |
| TO-236_AA | TO / Gull-Wing |  | TO-236_AA_with_gdt.stp TO-236_AA_with_gdt_single_solid.stp |
| MO-211_AA | BGA / Ball |  | MO-211_AA_with_gdt.stp MO-211_AA_with_gdt_single_solid.stp |
| MO-220_VEEB | QFN / Flat No-Lead Edge |  | MO-220_VEEB_with_gdt.stp MO-220_VEEB_with_gdt_single_solid.stp |
| 0402 CHIP | RESC / Rectangular End Cap |  | KOA_0402_1E_RK73B_with_gdt.stp KOA_0402_1E_RK73B_with_gdt_single_solid.stp |

Figure A1. JEDEC TO-236 Issue H.

TO-236_AA_with_gdt.stp

Figure A2. JEDEC DO-214 Issue D.

DO-214_AB_with_gdt_single_solid.stp

## APPENDIX B – PackageGdtReporter

To demonstrate the extraction of GD&T annotations from the provided physical files, a sample Java utility is provided that uses the JSDAI query routines to traverse an AP210 package model  and report the GD&T elements in a text-based format.

The following is the output obtained by executing the PackageGdtReporter  on the sample model ‘TO-236_AA_with_gdt.stp’. As may be seen in the output, the model contains three datums, numerous dimensions (both dimensional_size and dimensional_location, as well as both position and surface profile tolerances. For all GD&T elements, the associated faces from the solid models are reported, as well as their role in the body and/or terminals of the package.

Importing.... C:\Users\James Stori\Workspace\EclipseWorkspace\JSDAIexpress\P21 Files\TO-236_AA_with_gdt.stp

--- Exchange structure: C:\Users\James Stori\Workspace\EclipseWorkspace\JSDAIexpress\P21 Files\TO-236_AA_with_gdt.stp

--- Imported to the repository: TO_236_AA_annotation_pin_all_stp

--- Reading time=0sec

Datum: 'A'

FEATUREOFSIZE

Face: #146 of body

Face: #228 of body

Datum: 'B'

FEATUREOFSIZE

Face: #115 of body

Face: #353 of body

Datum: 'C'

TANGENTPLANE

Face: #670 of terminal 1

Face: #670 of terminal 2

Face: #670 of terminal 3

Dimensional size: 'E' : [ 2.64 MM (MAXIMUM) ][ 2.1 MM (MINIMUM) ]

FEATUREOFSIZE

Face: #646 of terminal 1

Face: #646 of terminal 2

Face: #646 of terminal 3

Dimensional size: 'A2' : [ 0.85 MM (NOMINAL) ][ 1.02 MM (MAXIMUM) ][ 0.7 MM (MINIMUM) ]

FEATUREOFSIZE

Face: #290 of body

Face: #53 of body

Composite Dimensional size: 'b' : [ 0.3 MM (MINIMUM) ][ 0.5 MM (MAXIMUM) ]

FEATUREOFSIZE

Face: #469 of terminal 1

Face: #781 of terminal 1

FEATUREOFSIZE

Face: #469 of terminal 2

Face: #781 of terminal 2

FEATUREOFSIZE

Face: #469 of terminal 3

Face: #781 of terminal 3

Dimensional size: 'D' : [ 3.04 MM (MAXIMUM) ][ 2.9 MM (NOMINAL) ][ 2.8 MM (MINIMUM) ]

FEATUREOFSIZE

Face: #146 of body

Face: #228 of body

Dimensional size: 'E1' : [ 1.3 MM (NOMINAL) ][ 1.2 MM (MINIMUM) ][ 1.4 MM (MAXIMUM) ]

FEATUREOFSIZE

Face: #115 of body

Face: #353 of body

Dimensional location: 'e' (BASIC) : [ 0.95 MM ]

Feature 1:

CENTERPLANE

Face: #469 of terminal 3

Face: #781 of terminal 3

Feature 2:

CENTERPLANE

Face: #469 of terminal 2

Face: #781 of terminal 2

Dimensional location: 'e1' (BASIC) : [ 1.9 MM ]

Feature 1:

CENTERPLANE

Face: #469 of terminal 1

Face: #781 of terminal 1

Feature 2:

CENTERPLANE

Face: #469 of terminal 2

Face: #781 of terminal 2

Dimensional location: 'A' : [ 1.17 MM (MAXIMUM) ][ 0.7 MM (MINIMUM) ]

Feature 1:

GENERIC

Face: #290 of body

Feature 2:

TANGENTPLANE

Face: #670 of terminal 1

Face: #670 of terminal 2

Face: #670 of terminal 3

Dimensional location: 'A1' : [ 0.15 MM (MAXIMUM) ][ 0.05 MM (MINIMUM) ]

Feature 1:

GENERIC

Face: #53 of body

Feature 2:

TANGENTPLANE

Face: #670 of terminal 1

Face: #670 of terminal 2

Face: #670 of terminal 3

Tolerance (POSITION): 'dim E center position' 0.15 MM

Datum System [C(MMR) A B ]

CENTERPLANE

Face: #646 of terminal 1

Face: #646 of terminal 2

Face: #646 of terminal 3

Tolerance (SURFACEPROFILE): 'terminal bottom surface profile' 0.1 MM

Datum System [C ]

TANGENTPLANE

Face: #670 of terminal 1

Face: #670 of terminal 2

Face: #670 of terminal 3

Composite Tolerance (POSITION): 'terminal centre plane position' 0.2 MM

CENTERPLANE

Face: #469 of terminal 1

Face: #781 of terminal 1

CENTERPLANE

Face: #469 of terminal 2

Face: #781 of terminal 2

CENTERPLANE

Face: #469 of terminal 3

Face: #781 of terminal 3

Done